ПРОВЕРЯЕМЫЙ ПРОГРАММНЫЙ МОДУЛЬ

# Энергетический регистр и верхняя граница D–D-петли

Полный исходный код и автоматические тесты · редакция 2.0-alpha-1

|  | Назначение Модуль вычисляет заведомо оптимистическую верхнюю границу K_loop,max. Если даже она меньше единицы, реальная система с потерями не может стать самоподдерживающейся за счёт D–D-продуктов при заданных n и τ. |
| --- | --- |

| Версия | 2.0-alpha-1 |
| --- | --- |
| Дата фиксации | 05.09.2026 |
| Автор проекта | Н. Н. Зобнин |
| Среда исполнения | Python 3; только стандартная библиотека для расчётного модуля |
| Основной файл | dd_loop_upper_bound_alpha1.py |
| SHA-256 основного файла | a0dace7422cd407f17504876a2f96a43f6f10ce5deb08b6fab5f1acd82db3761 |
| Файл тестов | test_dd_loop_upper_bound_alpha1.py |
| SHA-256 тестов | a765806031040f6290d6aedd5af77170e0a0d6c7a83caf23338a8d43e094dfcd |
| Итог тестирования | 6 тестов из 6 — успешно; 0,030 с в контрольном запуске |
| Машиночитаемые результаты | alpha1_summary.json; alpha1_table.csv |

# Как воспроизвести расчёт

Сохранить основной файл и файл тестов в одном каталоге.

Выполнить: python -m unittest -v test_dd_loop_upper_bound_alpha1.py

Выполнить: python dd_loop_upper_bound_alpha1.py --output-dir alpha1_results

Сверить решение с полем decision в alpha1_summary.json.

|  | Контрольное решение NO_GO_SELF_SUSTAINING_DD_AT_CURRENT_PARAMETERS; K_loop,max = 4.162312e-10 при 100 кэВ на дейтрон, встречной геометрии и 100%-ном возврате всей энергии. |
| --- | --- |

# Листинг 1. Расчётный модуль

| № | dd_loop_upper_bound_alpha1.py |
| --- | --- |
| 1 | #!/usr/bin/env python3 |
| 2 | """V2.0-alpha-1: strict upper bound for a D-D fusion feedback loop. |
| 3 |  |
| 4 | The module answers a deliberately narrow question: can energy released by |
| 5 | D-D reactions recreate the energetic-deuteron population that caused those |
| 6 | reactions? The default ``absolute_upper`` mode assumes 100% capture of all |
| 7 | fusion-product energy and 100% conversion into the next deuteron tail. It |
| 8 | therefore overestimates, never underestimates, the physically useful feedback. |
| 9 |  |
| 10 | Reference cross sections follow Bosch & Hale, Nuclear Fusion 32 (1992) 611, |
| 11 | DOI 10.1088/0029-5515/32/4/I07. Energies passed to the fit are centre-of-mass |
| 12 | energies in keV; the fit returns millibarns. |
| 13 |  |
| 14 | The code uses only the Python standard library and runs on a laptop. |
| 15 | """ |
| 16 |  |
| 17 | from __future__ import annotations |
| 18 |  |
| 19 | import argparse |
| 20 | import csv |
| 21 | import json |
| 22 | import math |
| 23 | from dataclasses import asdict, dataclass |
| 24 | from pathlib import Path |
| 25 | from typing import Iterable, Literal |
| 26 |  |
| 27 |  |
| 28 | ELEMENTARY_CHARGE_C = 1.602_176_634e-19 |
| 29 | DEUTERON_MASS_KG = 3.343_583_7768e-27 |
| 30 | KEV_TO_J = 1.0e3 * ELEMENTARY_CHARGE_C |
| 31 | MEV_TO_J = 1.0e6 * ELEMENTARY_CHARGE_C |
| 32 | MILLIBARN_TO_M2 = 1.0e-31 |
| 33 |  |
| 34 | Topology = Literal["beam_target", "head_on_equal_beams"] |
| 35 | ReturnMode = Literal["absolute_upper", "charged_products_only"] |
| 36 |  |
| 37 |  |
| 38 | @dataclass(frozen=True) |
| 39 | class BoschHaleBranch: |
| 40 | name: str |
| 41 | b_g: float |
| 42 | a: tuple[float, float, float, float, float] |
| 43 | b: tuple[float, float, float, float] |
| 44 | q_total_mev: float |
| 45 | q_charged_mev: float |
| 46 |  |
| 47 | def cross_section_m2(self, e_cm_kev: float) -> float: |
| 48 | """Return branch cross section in m^2 for E_cm in keV.""" |
| 49 | if e_cm_kev <= 0.0: |
| 50 | return 0.0 |
| 51 | e = e_cm_kev |
| 52 | a1, a2, a3, a4, a5 = self.a |
| 53 | b1, b2, b3, b4 = self.b |
| 54 | numerator = a1 + e * (a2 + e * (a3 + e * (a4 + e * a5))) |
| 55 | denominator = 1.0 + e * (b1 + e * (b2 + e * (b3 + e * b4))) |
| 56 | s_kev_millibarn = numerator / denominator |
| 57 | sigma_millibarn = (s_kev_millibarn / e) * math.exp(-self.b_g / math.sqrt(e)) |
| 58 | return max(0.0, sigma_millibarn * MILLIBARN_TO_M2) |
| 59 |  |
| 60 |  |
| 61 | DD_N_HE3 = BoschHaleBranch( |
| 62 | name="D(d,n)3He", |
| 63 | b_g=31.3970, |
| 64 | a=(5.3701e4, 3.3027e2, -1.2706e-1, 2.9327e-5, -2.5151e-9), |
| 65 | b=(0.0, 0.0, 0.0, 0.0), |
| 66 | q_total_mev=3.268, |
| 67 | q_charged_mev=0.820, |
| 68 | ) |
| 69 |  |
| 70 | DD_P_T = BoschHaleBranch( |
| 71 | name="D(d,p)T", |
| 72 | b_g=31.3970, |
| 73 | a=(5.5576e4, 2.1054e2, -3.2638e-2, 1.4987e-6, 1.8181e-10), |
| 74 | b=(0.0, 0.0, 0.0, 0.0), |
| 75 | q_total_mev=4.033, |
| 76 | q_charged_mev=4.033, |
| 77 | ) |
| 78 |  |
| 79 | DD_BRANCHES = (DD_N_HE3, DD_P_T) |
| 80 |  |
| 81 |  |
| 82 | @dataclass(frozen=True) |
| 83 | class EnergyLedger: |
| 84 | tail_inventory_j: float |
| 85 | nuclear_release_j: float |
| 86 | useful_feedback_j: float |
| 87 | unreturned_product_energy_j: float |
| 88 | numerical_residual_j: float |
| 89 |  |
| 90 | @property |
| 91 | def closure_fraction(self) -> float: |
| 92 | scale = max(abs(self.nuclear_release_j), 1.0e-300) |
| 93 | return abs(self.numerical_residual_j) / scale |
| 94 |  |
| 95 |  |
| 96 | @dataclass(frozen=True) |
| 97 | class LoopResult: |
| 98 | topology: str |
| 99 | return_mode: str |
| 100 | tail_energy_kev: float |
| 101 | e_cm_kev: float |
| 102 | density_m3: float |
| 103 | confinement_s: float |
| 104 | volume_m3: float |
| 105 | relative_speed_m_s: float |
| 106 | sigma_total_m2: float |
| 107 | sigma_v_m3_s: float |
| 108 | reaction_rate_m3_s: float |
| 109 | reactions_per_s: float |
| 110 | fusion_power_w: float |
| 111 | tail_inventory_j: float |
| 112 | useful_feedback_power_w: float |
| 113 | k_loop_max: float |
| 114 | required_n_tau_m3_s: float |
| 115 | required_confinement_s: float |
| 116 | energy_ledger: EnergyLedger |
| 117 |  |
| 118 |  |
| 119 | def centre_of_mass_energy_kev(tail_energy_kev: float, topology: Topology) -> float: |
| 120 | if topology == "beam_target": |
| 121 | return 0.5 * tail_energy_kev |
| 122 | if topology == "head_on_equal_beams": |
| 123 | return 2.0 * tail_energy_kev |
| 124 | raise ValueError(f"Unsupported topology: {topology}") |
| 125 |  |
| 126 |  |
| 127 | def pair_count_factor(topology: Topology) -> float: |
| 128 | """Factor in R/V = factor * n^2 * sigma*v for the idealized case. |
| 129 |  |
| 130 | beam_target treats n as both the dilute tail normalization and an |
| 131 | undepleted target density when forming an illustrative volume rate; its |
| 132 | per-tail loop coefficient is independent of tail fraction. The equal-beam |
| 133 | case uses the identical-particle 1/2 factor, which is already optimistic |
| 134 | because every encounter is assigned the head-on relative energy. |
| 135 | """ |
| 136 | return 1.0 if topology == "beam_target" else 0.5 |
| 137 |  |
| 138 |  |
| 139 | def relative_speed_m_s(e_cm_kev: float) -> float: |
| 140 | reduced_mass = 0.5 * DEUTERON_MASS_KG |
| 141 | return math.sqrt(2.0 * e_cm_kev * KEV_TO_J / reduced_mass) |
| 142 |  |
| 143 |  |
| 144 | def branch_quantities(e_cm_kev: float, return_mode: ReturnMode): |
| 145 | values = [] |
| 146 | for branch in DD_BRANCHES: |
| 147 | sigma = branch.cross_section_m2(e_cm_kev) |
| 148 | q_mev = branch.q_total_mev if return_mode == "absolute_upper" else branch.q_charged_mev |
| 149 | values.append((branch, sigma, q_mev * MEV_TO_J)) |
| 150 | return values |
| 151 |  |
| 152 |  |
| 153 | def calculate_loop_upper_bound( |
| 154 | tail_energy_kev: float, |
| 155 | density_m3: float, |
| 156 | confinement_s: float, |
| 157 | volume_m3: float, |
| 158 | topology: Topology = "head_on_equal_beams", |
| 159 | return_mode: ReturnMode = "absolute_upper", |
| 160 | feedback_efficiency: float = 1.0, |
| 161 | ) -> LoopResult: |
| 162 | """Calculate an optimistic one-cycle feedback-energy upper bound.""" |
| 163 | if tail_energy_kev <= 0.0: |
| 164 | raise ValueError("tail_energy_kev must be positive") |
| 165 | if density_m3 < 0.0 or confinement_s < 0.0 or volume_m3 <= 0.0: |
| 166 | raise ValueError("density/confinement must be nonnegative and volume positive") |
| 167 | if not 0.0 <= feedback_efficiency <= 1.0: |
| 168 | raise ValueError("feedback_efficiency must be between 0 and 1") |
| 169 |  |
| 170 | e_cm = centre_of_mass_energy_kev(tail_energy_kev, topology) |
| 171 | speed = relative_speed_m_s(e_cm) |
| 172 | branches = branch_quantities(e_cm, return_mode) |
| 173 | sigma_total = sum(sigma for _, sigma, _ in branches) |
| 174 | sigma_v_total = sigma_total * speed |
| 175 | factor = pair_count_factor(topology) |
| 176 |  |
| 177 | rate_density = factor * density_m3**2 * sigma_v_total |
| 178 | reactions_per_s = rate_density * volume_m3 |
| 179 | total_fusion_power = sum( |
| 180 | factor * density_m3**2 * sigma * speed * volume_m3 * branch.q_total_mev * MEV_TO_J |
| 181 | for branch, sigma, _ in branches |
| 182 | ) |
| 183 | if return_mode == "absolute_upper": |
| 184 | # Reuse the identical arithmetic path so eta=1 closes exactly. |
| 185 | useful_feedback_power = feedback_efficiency * total_fusion_power |
| 186 | else: |
| 187 | useful_feedback_power = feedback_efficiency * sum( |
| 188 | factor * density_m3**2 * sigma * speed * volume_m3 * q_return_j |
| 189 | for _, sigma, q_return_j in branches |
| 190 | ) |
| 191 |  |
| 192 | tail_inventory = density_m3 * volume_m3 * tail_energy_kev * KEV_TO_J |
| 193 | useful_feedback_energy = useful_feedback_power * confinement_s |
| 194 | nuclear_release = total_fusion_power * confinement_s |
| 195 | unreturned = nuclear_release - useful_feedback_energy |
| 196 | if abs(unreturned) < 1.0e-14 * max(abs(nuclear_release), 1.0e-300): |
| 197 | unreturned = 0.0 |
| 198 | residual = nuclear_release - useful_feedback_energy - unreturned |
| 199 | ledger = EnergyLedger( |
| 200 | tail_inventory_j=tail_inventory, |
| 201 | nuclear_release_j=nuclear_release, |
| 202 | useful_feedback_j=useful_feedback_energy, |
| 203 | unreturned_product_energy_j=unreturned, |
| 204 | numerical_residual_j=residual, |
| 205 | ) |
| 206 | k_loop = useful_feedback_energy / tail_inventory if tail_inventory > 0.0 else 0.0 |
| 207 |  |
| 208 | weighted_q_return = sum(sigma * q for _, sigma, q in branches) |
| 209 | gain_per_n_tau = factor * speed * weighted_q_return / (tail_energy_kev * KEV_TO_J) |
| 210 | gain_per_n_tau *= feedback_efficiency |
| 211 | required_n_tau = math.inf if gain_per_n_tau <= 0.0 else 1.0 / gain_per_n_tau |
| 212 | required_confinement = math.inf if density_m3 <= 0.0 else required_n_tau / density_m3 |
| 213 |  |
| 214 | return LoopResult( |
| 215 | topology=topology, |
| 216 | return_mode=return_mode, |
| 217 | tail_energy_kev=tail_energy_kev, |
| 218 | e_cm_kev=e_cm, |
| 219 | density_m3=density_m3, |
| 220 | confinement_s=confinement_s, |
| 221 | volume_m3=volume_m3, |
| 222 | relative_speed_m_s=speed, |
| 223 | sigma_total_m2=sigma_total, |
| 224 | sigma_v_m3_s=sigma_v_total, |
| 225 | reaction_rate_m3_s=rate_density, |
| 226 | reactions_per_s=reactions_per_s, |
| 227 | fusion_power_w=total_fusion_power, |
| 228 | tail_inventory_j=tail_inventory, |
| 229 | useful_feedback_power_w=useful_feedback_power, |
| 230 | k_loop_max=k_loop, |
| 231 | required_n_tau_m3_s=required_n_tau, |
| 232 | required_confinement_s=required_confinement, |
| 233 | energy_ledger=ledger, |
| 234 | ) |
| 235 |  |
| 236 |  |
| 237 | def logarithmic_grid(low: float, high: float, count: int) -> list[float]: |
| 238 | if low <= 0.0 or high <= low or count < 2: |
| 239 | raise ValueError("invalid logarithmic grid") |
| 240 | step = math.log(high / low) / (count - 1) |
| 241 | return [low * math.exp(i * step) for i in range(count)] |
| 242 |  |
| 243 |  |
| 244 | def scan_strict_bound( |
| 245 | density_m3: float, |
| 246 | confinement_s: float, |
| 247 | volume_m3: float, |
| 248 | energy_min_kev: float = 0.2, |
| 249 | energy_max_kev: float = 100.0, |
| 250 | grid_points: int = 2001, |
| 251 | ) -> tuple[LoopResult, list[LoopResult]]: |
| 252 | results: list[LoopResult] = [] |
| 253 | for topology in ("beam_target", "head_on_equal_beams"): |
| 254 | for energy in logarithmic_grid(energy_min_kev, energy_max_kev, grid_points): |
| 255 | results.append( |
| 256 | calculate_loop_upper_bound( |
| 257 | energy, |
| 258 | density_m3, |
| 259 | confinement_s, |
| 260 | volume_m3, |
| 261 | topology=topology, |
| 262 | return_mode="absolute_upper", |
| 263 | feedback_efficiency=1.0, |
| 264 | ) |
| 265 | ) |
| 266 | return max(results, key=lambda item: item.k_loop_max), results |
| 267 |  |
| 268 |  |
| 269 | def result_to_dict(result: LoopResult) -> dict: |
| 270 | data = asdict(result) |
| 271 | return data |
| 272 |  |
| 273 |  |
| 274 | def write_outputs(output_dir: Path, summary: dict, rows: Iterable[LoopResult]) -> None: |
| 275 | output_dir.mkdir(parents=True, exist_ok=True) |
| 276 | with (output_dir / "alpha1_summary.json").open("w", encoding="utf-8") as stream: |
| 277 | json.dump(summary, stream, ensure_ascii=False, indent=2, allow_nan=False) |
| 278 |  |
| 279 | fieldnames = [ |
| 280 | "topology", "return_mode", "tail_energy_kev", "e_cm_kev", "density_m3", |
| 281 | "confinement_s", "volume_m3", "sigma_total_m2", "sigma_v_m3_s", |
| 282 | "reactions_per_s", "fusion_power_w", "tail_inventory_j", |
| 283 | "useful_feedback_power_w", "k_loop_max", "required_n_tau_m3_s", |
| 284 | "required_confinement_s", |
| 285 | ] |
| 286 | with (output_dir / "alpha1_table.csv").open("w", encoding="utf-8-sig", newline="") as stream: |
| 287 | writer = csv.DictWriter(stream, fieldnames=fieldnames) |
| 288 | writer.writeheader() |
| 289 | for row in rows: |
| 290 | raw = result_to_dict(row) |
| 291 | writer.writerow({key: raw[key] for key in fieldnames}) |
| 292 |  |
| 293 |  |
| 294 | def run_default_study(output_dir: Path) -> dict: |
| 295 | density = 1.62e16 |
| 296 | confinement = 20.0e-6 |
| 297 | radius = 3.259053e-3 |
| 298 | length = 2.998427 |
| 299 | volume = math.pi * radius**2 * length |
| 300 | energies = (0.2, 1.0, 3.0, 10.0, 30.0, 100.0) |
| 301 |  |
| 302 | strict_best, _ = scan_strict_bound(density, confinement, volume) |
| 303 | table_rows = [] |
| 304 | for topology in ("beam_target", "head_on_equal_beams"): |
| 305 | for energy in energies: |
| 306 | table_rows.append( |
| 307 | calculate_loop_upper_bound( |
| 308 | energy, density, confinement, volume, topology=topology, |
| 309 | return_mode="absolute_upper", feedback_efficiency=1.0, |
| 310 | ) |
| 311 | ) |
| 312 |  |
| 313 | time_sensitivity = [] |
| 314 | for tau in (121e-9, 20e-6, 1e-3, 1.0): |
| 315 | time_sensitivity.append( |
| 316 | calculate_loop_upper_bound( |
| 317 | 100.0, density, tau, volume, |
| 318 | topology="head_on_equal_beams", return_mode="absolute_upper", |
| 319 | feedback_efficiency=1.0, |
| 320 | ) |
| 321 | ) |
| 322 |  |
| 323 | density_sensitivity = [] |
| 324 | for n in (1.62e16, 3.0e16, 1.0e18, 1.0e20): |
| 325 | density_sensitivity.append( |
| 326 | calculate_loop_upper_bound( |
| 327 | 100.0, n, confinement, volume, |
| 328 | topology="head_on_equal_beams", return_mode="absolute_upper", |
| 329 | feedback_efficiency=1.0, |
| 330 | ) |
| 331 | ) |
| 332 |  |
| 333 | charged_only = calculate_loop_upper_bound( |
| 334 | 100.0, density, confinement, volume, |
| 335 | topology="head_on_equal_beams", return_mode="charged_products_only", |
| 336 | feedback_efficiency=1.0, |
| 337 | ) |
| 338 |  |
| 339 | summary = { |
| 340 | "module": "2.0-alpha-1", |
| 341 | "model_status": "strict optimistic upper bound; not a self-consistent plasma simulation", |
| 342 | "default_parameters": { |
| 343 | "density_m3": density, |
| 344 | "confinement_s": confinement, |
| 345 | "radius_m": radius, |
| 346 | "length_m": length, |
| 347 | "volume_m3": volume, |
| 348 | "energy_range_kev": [0.2, 100.0], |
| 349 | }, |
| 350 | "strict_best": result_to_dict(strict_best), |
| 351 | "charged_products_only_at_best_energy": result_to_dict(charged_only), |
| 352 | "time_sensitivity": [result_to_dict(item) for item in time_sensitivity], |
| 353 | "density_sensitivity": [result_to_dict(item) for item in density_sensitivity], |
| 354 | "decision": ( |
| 355 | "NO_GO_SELF_SUSTAINING_DD_AT_CURRENT_PARAMETERS" |
| 356 | if strict_best.k_loop_max < 1.0 else "CONTINUE_REFINEMENT" |
| 357 | ), |
| 358 | } |
| 359 | write_outputs(output_dir, summary, table_rows) |
| 360 | return summary |
| 361 |  |
| 362 |  |
| 363 | def _safe_json_value(value): |
| 364 | if isinstance(value, float) and not math.isfinite(value): |
| 365 | return None |
| 366 | if isinstance(value, dict): |
| 367 | return {k: _safe_json_value(v) for k, v in value.items()} |
| 368 | if isinstance(value, list): |
| 369 | return [_safe_json_value(v) for v in value] |
| 370 | return value |
| 371 |  |
| 372 |  |
| 373 | def main() -> None: |
| 374 | parser = argparse.ArgumentParser(description=__doc__) |
| 375 | parser.add_argument("--output-dir", type=Path, default=Path("alpha1_results")) |
| 376 | args = parser.parse_args() |
| 377 | summary = _safe_json_value(run_default_study(args.output_dir)) |
| 378 | print(json.dumps(summary, ensure_ascii=False, indent=2)) |
| 379 |  |
| 380 |  |
| 381 | if __name__ == "__main__": |
| 382 | main() |

# Листинг 2. Автоматические тесты

| № | test_dd_loop_upper_bound_alpha1.py |
| --- | --- |
| 1 | import math |
| 2 | import tempfile |
| 3 | import unittest |
| 4 | from pathlib import Path |
| 5 |  |
| 6 | from dd_loop_upper_bound_alpha1 import ( |
| 7 | DD_N_HE3, |
| 8 | DD_P_T, |
| 9 | calculate_loop_upper_bound, |
| 10 | run_default_study, |
| 11 | scan_strict_bound, |
| 12 | ) |
| 13 |  |
| 14 |  |
| 15 | class Alpha1Tests(unittest.TestCase): |
| 16 | def test_bosch_hale_reference_points(self): |
| 17 | total_10_kev_mb = ( |
| 18 | DD_N_HE3.cross_section_m2(10.0) + DD_P_T.cross_section_m2(10.0) |
| 19 | ) / 1.0e-31 |
| 20 | total_100_kev_mb = ( |
| 21 | DD_N_HE3.cross_section_m2(100.0) + DD_P_T.cross_section_m2(100.0) |
| 22 | ) / 1.0e-31 |
| 23 | self.assertAlmostEqual(total_10_kev_mb, 0.5591251985, places=8) |
| 24 | self.assertAlmostEqual(total_100_kev_mb, 70.04894814, places=6) |
| 25 |  |
| 26 | def test_zero_density_has_zero_gain(self): |
| 27 | result = calculate_loop_upper_bound(100.0, 0.0, 20e-6, 1e-4) |
| 28 | self.assertEqual(result.k_loop_max, 0.0) |
| 29 | self.assertEqual(result.fusion_power_w, 0.0) |
| 30 |  |
| 31 | def test_energy_ledger_closes(self): |
| 32 | result = calculate_loop_upper_bound(100.0, 1.62e16, 20e-6, 1e-4) |
| 33 | self.assertLess(result.energy_ledger.closure_fraction, 1e-14) |
| 34 |  |
| 35 | def test_required_n_tau_reconstructs_unity(self): |
| 36 | base = calculate_loop_upper_bound(100.0, 1.62e16, 20e-6, 1e-4) |
| 37 | check = calculate_loop_upper_bound( |
| 38 | 100.0, 1.0, base.required_n_tau_m3_s, 1e-4, |
| 39 | topology="head_on_equal_beams", |
| 40 | ) |
| 41 | self.assertTrue(math.isclose(check.k_loop_max, 1.0, rel_tol=1e-12)) |
| 42 |  |
| 43 | def test_strict_scan_maximum_is_upper_boundary(self): |
| 44 | best, _ = scan_strict_bound(1.62e16, 20e-6, 1e-4, grid_points=101) |
| 45 | self.assertEqual(best.topology, "head_on_equal_beams") |
| 46 | self.assertTrue(math.isclose(best.tail_energy_kev, 100.0, rel_tol=1e-12)) |
| 47 |  |
| 48 | def test_default_study_writes_reproducible_outputs(self): |
| 49 | with tempfile.TemporaryDirectory() as tmp: |
| 50 | summary = run_default_study(Path(tmp)) |
| 51 | self.assertTrue((Path(tmp) / "alpha1_summary.json").is_file()) |
| 52 | self.assertTrue((Path(tmp) / "alpha1_table.csv").is_file()) |
| 53 | self.assertEqual(summary["decision"], "NO_GO_SELF_SUSTAINING_DD_AT_CURRENT_PARAMETERS") |
| 54 |  |
| 55 |  |
| 56 | if __name__ == "__main__": |
| 57 | unittest.main() |

# Контроль целостности

Приведённые выше контрольные суммы относятся к байтовому содержимому файлов на дату фиксации. Любое изменение кода требует новой версии, повторного тестирования и новых SHA-256.